Project By:

EWB (ENGINEERS WITH BORDERS)

Project Title

Smart Cashier System (AI-Powered Checkout with Weight Verification)

Project Type

University + Personal project (Team project)

Team

4 students

My Role:

Mohammed Basheer Al-oshari:

- Mechanical design & assembly lead

- Electrical wiring & power/control circuits

- ~30% contribution to software/control, assisting a teammate in code implementation

- System integration + troubleshooting

Problem Statement

Traditional cashier checkout causes:

- Long waiting time and congestion during peak hours

- Dependence on a human cashier, increasing monthly operating costs

- Higher risk of manual entry errors

Proposed Solution

A Smart Cashier system that:

- Uses Machine Learning (Computer Vision) to recognize products automatically

- Uses a weight sensor (Load Cell) to verify the recognized product and reduce mistakes/fraud

- Controls a conveyor belt to automate the checkout flow

- Communicates between a PC (AI processing) and an ESP32 controller via Wi-Fi (HTTP)

System Description

Mechanical Design

- A cashier frame structure with a conveyor belt

- Single-phase induction motor with a 10:1 gearbox to achieve an appropriate belt speed

- Aesthetic enclosure to hide internal components and improve appearance

- CAD design performed using SolidWorks (images will be included)

Electrical Design

- Motor movement control: forward / backward / stop

- Converting controller signals into motor actions using:

  - Relays

  - Transistors

- Stable power distribution for controller and actuators

Control & Software

- ESP32 as the main controller (receives commands and controls motor logic)

- PC-side AI processing:

  - Python + OpenCV

  - TensorFlow Lite model for classification

- Weight verification module (Load Cell + amplifier e.g., HX711) as product validation layer

Software Implementation (Based on Code)

Real-time Recognition (Python)

The system uses a TFLite model to classify products from camera frames:

- Preprocessing follows Teachable Machine style normalization (scale to [-1, 1])

- Confidence threshold = 0.85 for valid detections

- Stable frames logic: the same label must appear consistently for multiple frames before sending to ESP32

- Cooldown time: prevents sending repeated commands too frequently

- Sends product name to ESP32 using HTTP request:

  - GET http://ESP32_IP/post?product=<label>

Model Inspection Script

A utility script checks the TFLite model input/output shapes and data types to ensure compatibility with runtime.

Labels File

A labels.txt file maps model output index to product names.

Wi-Fi Test Scripts

Simple Python scripts to validate ESP32 communication and server endpoints.

Results

What Worked

- The main concept was validated:

  - Product recognition worked

  - PC-to-ESP32 communication worked

- Core idea proved feasible under limited time

Limitations / Issues

- Conveyor/motor/servo reliability issues (unexpected stopping)

- Weight calibration was difficult under a moving belt:

  - Solution: added a second scale at the end of the basket

- Lighting was not suitable, reducing recognition reliability

- “Door opens after payment” feature was not implemented due to time constraints

Lessons Learned

- Computer vision systems are highly sensitive to lighting and camera positioning

- Load cells require stable mounting and careful calibration, especially with moving systems

- Integration (mechanical + electrical + software) needs time for testing and iteration

- Stable-decision logic (stable frames + cooldown) is essential to reduce false triggers

Future Improvements

- Fully enclosed “boxed” design to control lighting and reduce theft risk

- Implement payment workflow with an electronic wallet

- Add a real locking/unlocking door mechanism that opens only after payment confirmation

- Improve motor control:

  - Prefer DC motor + encoder (closed-loop control) for stability

- Add better illumination (diffused LEDs) for consistent vision performance

- Add product database + pricing and UI dashboard for a real cashier experience

Project Summary:

Smart Cashier System is a mechatronics project integrating mechanical design, electrical control, and AI-based vision to automate checkout, reduce congestion, and lower operational costs. Despite time constraints and calibration/lighting challenges, the core system worked and offers strong potential for real-world refinement.